# Solver records and temporal data checks

This summary documents the solver termination records for the 13 archived pathway results and the temporal indexing checks for the representative-week data. Its scope is computational provenance and data indexing. These checks do not certify physical feasibility of dispatch or provide a complete validation of the model.

# Archived solver performance

The archived runs used Gurobi Optimizer 12.0.3 with a target relative MIP gap of 1%. Eight runs reached that tolerance; five terminated at the time limit with an incumbent solution. The table reports the achieved gap for each run. The archived results have not been reoptimized for this summary.

| Pathway | Constraints | Variables | Time (s) | Gap (%) | Termination |
| --- | --- | --- | --- | --- | --- |
| GTL-BH | 400,022 | 352,940 | 65.01 | 0.9087 | Gap met |
| GTL | 60,768 | 49,845 | 0.45 | 0.3280 | Gap met |
| GTL-GH | 537,560 | 472,775 | 3,601.86 | 27.2950 | Time limit |
| CTL-BH | 137,065 | 79,086 | 23.19 | 0.5662 | Gap met |
| CTL | 206,605 | 138,306 | 524.80 | 0.9807 | Gap met |
| CCU-BH-FT | 217,142 | 155,671 | 82.09 | 0.8932 | Gap met |
| CCU-BH-MTJ | 217,142 | 155,671 | 395.01 | 0.8579 | Gap met |
| CCU-GH-FT | 327,204 | 237,751 | 104.08 | 0.3246 | Gap met |
| CCU-GH-MTJ | 327,204 | 237,751 | 91.98 | 0.2252 | Gap met |
| DAC-BH-FT | 196,728 | 157,230 | 3,600.43 | 4.1714 | Time limit |
| DAC-BH-MTJ | 196,728 | 157,230 | 3,600.75 | 3.3594 | Time limit |
| DAC-GH-FT | 325,960 | 274,725 | 3,602.59 | 3.5338 | Time limit |
| DAC-GH-MTJ | 325,960 | 274,725 | 3,601.49 | 1.3769 | Time limit |

Gap met denotes termination within the specified 1% MIP tolerance. Time limit denotes an available incumbent without a 1% optimality certificate. The largest reported gap is 27.2950% for GTL-GH. MIP optimality gaps quantify objective-value uncertainty relative to the solver bound; they do not measure physical balance errors.

Source: solver_run_evidence.csv, supplied with the supplementary data. The file records the source log, log lines, solution timestamp, objective value and best bound for each pathway.

# Data indexing and record checks

The comparative horizon comprises weeks 1, 5, 14 and 44 of the representative year: 672 represented hours and 224 three-hour decision periods. Weekly demand and transport decisions are associated with four model weeks. The checks below address the identity and indexing of these records.

| Check | Evidence and scope | Result |
| --- | --- | --- |
| Run and result association | Cross-check the archived solution against its recorded objective, termination code and runtime. | 13 of 13 matched |
| Scenario coverage | Check for one uniquely identified result and solver record for each retained pathway. | 13 distinct pathways |
| Time dimensions | Check the selected horizon: 4 weeks × 168 h; 672 h ÷ 3 h per period. | 672 h and 224 periods |
| Resource-window association | For CCU-GH-FT, compare archived mean output for each resource record with the reconstructed selected 672-hour window. | 109 of 109 matched |

The resource-window check uses relative and absolute numerical tolerances of 10⁻⁹ for the mean-value comparison. It establishes agreement between archived and reconstructed records under the same input convention. Physical-unit interpretation, energy and material balances, and engineering feasibility are outside the certification scope of this summary.

# Traceability and interpretation

The accompanying evidence index links each pathway to its archived output and solver-log location. For CCU-GH-FT, archived_temporal_mapping_check.csv provides the resource-window comparisons, and the temporal mapping script records the selected-week indices. These records allow the reported termination statistics and indexing checks to be inspected without presenting every production decision in the supplement.

Solver termination, archive consistency and physical validation are separate assessments. The results reported here support the first two within the stated scope. A time-limited result retains its reported gap, and no global statement that all validation checks have passed is inferred from this summary.